Faculty of Information Technology

CNB2124 Data Communication and Computer Networking

Section 2

Term Project – Storage Area Network (SAN)

Date of submission– 10 July 2001

Prepared for:

En. Mohd Azul Mohamad Salleh

Ahmad Nazly Ahmad Fuad

UB1-9909-01311

TABLE OF CONTENTS

Introduction – Rapid evolution of computer storage capacity	3

What is SAN?	3

SNIA definition of SAN	3

SAN: Server-centric or storage-centric?	4

Server-centric	4

Storage-centric	4

SAN architecture and topology	4

SAN components	6

Interface	6

Interconnect	6

Fabric	6

Fibre Channel	6

SAN characteristics	7

Availability	7

Scalability	7

Performance	7

Connectivity	8

Manageability	8

Interoperability	8

Security	8

End-to-end support	8

SAN Advantages	8

SAN disadvantages	9

SAN applications in today’s world	10

Conclusion	11

Appendixes	11

References / bibliography	12

# Introduction – Rapid evolution of computer storage capacity

Computer storage capacity has increased rapidly mainly due to the unstoppable demands of external storage or secondary storage by organizations that employ the centralized management and also to the huge volume of data moving between transaction system and enterprises data storage. The significance of this scenario can be observed through the increasing of awareness on factors such as continuous availability, ease of management, scalability and resource sharing in diverse environment. The introduction of new specifically network architectures such as Storage Area Network (SAN) gives organizations an alternative, as it is being considered as the possible choice for the stated scenario and its significance. Experts believe that Storage Area Network is about to revolutionize the computer storage industry.

# What is SAN?

The proper definition of Storage Area Network (SAN) is a dedicated high-speed network that allows fast, reliable access among servers and external or independent storage resources. Specifically, it interconnects different kinds of data storage devices with associated data servers on behalf of a larger network of users and allows these storage devices to be shared among multiple hosts without affecting system performance. The most important component is that these storage devices are interfaced to the network through components like switches that provide dynamic any-server to any-storage solutions.

Storage Area Network actually makes a paradigm shift from the conventional methods of network storage where all the subsystems, software and hardware related to storage are moved off the server and take the form of a separate sub-network within organizations network structure. This network can use existing communication technology such as IBM’s optical fiber ESCON or it may use the newer Fibre Channel technology.

## SNIA definition of SAN

According to Storage Networking Industry Association (SNIA), the primary purpose of Storage Area Network is the transfer of data between computer systems and storage elements and among storage elements. A Storage Area Network consists of a communication infrastructure, which provides physical connections, and a management layer, which organizes the connections, storage elements, and computer systems so that the data transfer is secure and robust. A storage system consists of storage elements, storage devices, computer systems, plus applications and all control software, communicating over a network. Nevertheless, the SNIA definition specifically does not recognize the term SAN with Fibre Channel technology. When the term SAN is used in correlation with Fibre Channel technology, it is preferable to use term “Fibre Channel SAN”.

# SAN: Server-centric or storage-centric?

Briefly, there are two general methods used to set up a base network, which are server-centric and storage-centric. It is like centralized-server-based method and centralized-storage-based method.

## Server-centric

Server-centric method refers to the storage sub-systems such as hard disk arrays tape storage and optical storage that are directly connected to the servers through a high-speed interface such as SCSI. This is the suitable method of implementing storage for small organizations and networks. It shows that about 10 percent of the total storage in the organization is connected to the server and the remaining 90 percent is connected to different servers namely as file servers. These file servers are connected directly to the Local Area Network (LAN). The costs and performances for this method are unpredictable depend a lot on the configuration and the technologies used in a particular system.

## Storage-centric

The other method is the storage-centric. This is where mostly the Storage Area Network architecture is implemented. In this method, about 10 percent of the storage is connected to the server and the remaining 90 percent is connected to the rest of the network using a high-speed technology such as Fibre Channel.

# SAN architecture and topology

Storage Area Network architecture operates in a way that constructs all storage devices available to all servers on a Local Area Network (LAN) or Wide Area Network (WAN). As more storage devices are added to a SAN, they too will be accessible from any server in the larger network. The server acts only as a pathway between the end user and the stored data thus allowing network administrators to independently scale the storage or server processing power as requirements demand.

A SAN also allows numerous servers to access the same data so that repetition of information can be reduced, and allows data backup to occur directly over storage channels, reducing the bottleneck of the relatively slow LAN. Data is also more reliably available, as the breakdown of a single server will not cut off any storage from remaining servers.

STORAGE AREA NETWORK TOPOLOGY

ANOTHER STORAGE AREA NETWORK TOPOLOGY

All networked servers share storage devices as peer resources, which means that they are not the restricted property of any one server. SAN can be used to connect servers to storage, servers to each other, and storage to storage through hubs, switches and routers. A SAN carries only Input/Output (I/O) traffic between servers and storage devices and no general-purpose traffic such as electronic mail or other end-user applications are carried. This will prevent from any congestion in using a single network for all applications.

SAN use a various mixture of technologies, which include IBM’s Enterprise Systems Connection (ESCON), Fiber Distributed Data Interface (FDDI), Asynchronous Transfer Mode (ATM), IBM’s Serial Storage Architecture (SSA), and Fibre Channel. SAN architectures also let particular users or administrators to use a number of basic protocols such as TCP/IP and variations of SCSI. The popular implementation of SAN for open system is based on SCSI over Fibre Channel.

Storage Area Networks support disk mirroring, backup and restore, archival and retrieval of archived data, data migration from a storage device to another, and the sharing of data among different servers in a network.

# SAN components

There are three main components in a Storage Area Network, which are Interface, Interconnect and Fabric.

## Interface

The Interface allows storage to be external from the server and allows server clustering. The common SAN Interfaces are SCSI, Fibre Channel and other protocols.

## Interconnect

The Interconnect is the mechanism that multiple devices exchange data. Devices such as multiplexers, hubs, routers, gateways and switchers are used to link various Interfaces to Fabrics.

## Fabric

The Fabric is the platform based on switched SCSI, switched Fibre Channel or etc. The platform is the combination of network protocol and network topology. The use of gateways allows the SAN to be extended across WANs.

## Fibre Channel

Fibre Channel has been used as an alternative to SCSI in generating high-speed links among network devices. This type of Interface was developed by ANSI in the early 1990s, purposely as a means to transfer large amounts of data at faster rates. Fibre Channel is compatible with SCSI, IP, IEEE, 802.2, ATM Adaptation Layer for computer data, and Link Encapsulation, and it can be used over copper cabling or fiber-optic cable. Fibre Channel supports data rates of 133 Megabytes per second (Mbps), 266 Mbps, 532 Mbps and 1.0625 Gigabytes per second. In the near future, most probably there will be data rates of 4 Gigabytes per second. Currently, this technology supports distances of up to 10 kilometers, which indeed is a good choice for disaster recovery.

It is known that installing a Storage Area Network does not essentially mean organizations will have to change all their storage hardware. SCSI-to-Fibre Channel converters allow most accessible storage subsystems to be included into a SAN.

# SAN characteristics

A SAN storage resource needs to be served by, but subject to a flexible, centralized management system with characteristics like smoothly scalable capacity allocations, transparent to the client operating system, capable of integrating legacy storage, adaptable to any SAN infrastructure, adaptable to the use of RAID storage, compatible with operating system security requirements, capable of providing full data redundancy, enhances application performance, minimal or no client-side software or driver, enhances high-availability of storage and data, no proprietary hardware requirements and open system software.

Such characteristics can be ‘simplified’ into eight main features, which are availability, scalability, performance, connectivity, manageability, connectivity, interoperability and end-to-end support.

## Availability

Users or administrators concern most on full data and application availability. Data availability involves serviceability, component monitoring, automatic fault detection, isolation and recovery, online repairability, and complete system restoration after fixing a failure, beyond system and path redundancy. Application availability means having a detailed testing program that includes fast recovery from unexpected conditions caused by system component failures. To wrap it all up, SAN will achieve these goals through multiple levels of path and system redundancy and powerful backup and disaster-recovery software.

## Scalability

This feature lets growth in capacity, performance, connectivity, and availability without involving continuing operations. A SAN configuration is expected to grow without adding complexity and without reduction on performance. Scalability comprises of adaptability and portability for applications.

## Performance

SAN gives improvements on several levels of performance such as combining multiple simultaneous data paths, load-balance software, and dynamic path reallocation. There are other potential performance improvements but it relate to the design of admission-control mechanisms such as assuring bandwidth for high-priority data streams and time-dependant events. For overall performance enhancements, resource sharing, data sharing and Storage Resource Management (SRM) are used.

## Connectivity

In a SAN environment, every network server can deal with all the storage on the network. This level of connectivity will allow sharing business-critical data between heterogeneous or diverse platforms.

## Manageability

In administering networks, managing storage is one of the most significant costs. Increased manageability has become a main concern in all distributed network environments. Data management and SRM automate the processes of backup, restoration, configuration, monitoring, load balancing, diagnosing, and reporting. The degree to which a SAN can mix and automate some or all of these steps will decide its position in hierarchy of product classes.

## Interoperability

A SAN contains many components, and the lack of recognized standards has lifted the interoperability issue to a high level of concern. The condition is for all these elements to dialog and mix pleasantly, to allow the user the greatest flexibility in the choice of products and vendors. The existing Fibre Channel standards are a prerequisite but are not sufficient to attain interoperability among devices from various vendors. In the short-term period, users will need vendors to guarantee and maintain interoperability. Storage Networking Industry Association (SNIA) is currently defining standards to ease this problem.

## Security

SAN management must set in several levels of security to ensure complete access. So far, implementations that deal with access control are done through zoning and partitioning techniques.

## End-to-end support

Switching to SAN from previous architecture is an important task. It is a task that most users or administrators are not prepared to handle. That is why vendors must help to plan, integrate, install and service the entire SAN infrastructure.

# SAN Advantages

There are many advantages of implementing Storage Area Network. The main advantage is that storage is centrally located within the organization or enterprise. For large networks, particularly where there are regularly about tens or hundreds of servers present with hundreds of clients, having a SAN serves to streamline and simplify a host of tasks which is the applications and data that are centrally located, upgrading infrastructures and storage space is simplified, repetitive routines like backups and application updates are easier and overall, network performance is improved.

Then, SAN also gives superior connectivity. In a SAN environment, every server on the network can address all the storage on the network, for distance up to 10 kilometers with Fibre Channel support. Furthermore, any storage device can communicate to any other storage device on the network. Such potential enables the utilization of existing storage resources that may be spread throughout an organization, and encourages wider implementation of remote vaulting, mirroring, and clustering solutions.

SAN allows users or administrators to virtually centralized storage. Administrators can use SAN to segregate physical storage devices from the virtual presentation that normal users or clients see. This segregation provides a high level of flexibility in issuing and reorganizing resources. As it already mentioned before, storage on a SAN is shared, which resulting in centralized management, better utilization of resources, and improved enterprise-wide data management and protection.

VIRTUALLY CENTRALIZED STORAGE

No other network architectures can match the degree of scalability that SAN produces. SAN completely increases performance. The storage capacity can grow, its performance can scale along with its topology, and its availability can increase by providing redundant systems and data paths. All these degree are done without involving continuing operations.

# SAN disadvantages

Regardless of many advantages being mentioned, there are also some limitations or disadvantages in SAN technology. Although the concept has been known for quite some time but the implementation still new to most of organizations. The main limitation is the lack of interoperability between SAN fiber networks from different vendors. Software tools or utilities that are distributed with SAN hardware are limited to managing a single vendor’s SAN components. The idea of merging Fibre Channels switches from multiple vendors is not currently possible although current standard efforts; interoperability testing and multivendor development projects are produced to overcome these obstacles within the next 12 months.

That is why most ‘corporate’ SAN operations are limited to specific applications or functions such as LAN-free backup, serverless backup, disaster recovery, and storage consolidation or storage merging in Windows NT and UNIX environments.

# SAN applications in today’s world

Nowadays, Storage Area Networks (SANs) are considered as a vital part of any up to date High-Availability plan. An Internet Service Provider (ISP), a larger financial services enterprise, an educational institution, a government deparment or laboratory, a digital video production group, a television-broadcasting station moving to digital services, or any organization with increasing data storage needs are the main parties that need SANs.

Storage Area Networks may enable companies or organizations to operate in a business sense in a fundamentally new way, or in a way they have aimed to in recent years but the technology had really not been up to date or up to the challenge. Such organizations are usually geographically dissolved and depend upon timely and accurate information as the main corporate asset. Data is being considered as the ‘life-blood’ of the business. Today, the data required is stored in multiple, separated locations.

The information needed by these organizations may be contained in huge data storage but is more probably to be in the form of a combination of widely separated sources. These might include databases, email servers; but the huge amount of information is still held as normal files on the file systems. A high performance access to data for all authorized users across the business is needed to enable businesses to operate with a wide-open shared data concept.

Another business condition is that the information is always available. ‘Always’ here does not mean 99.999% system availability but it means 100% availability by users and their applications to the data.   A Storage Area Network provides essentially very high availability by the use of automatic data redundancy, automatic backups and the maintenance of nearby disaster recovery copies. Clustered servers with shared access to the data that can enthusiastically switch users and applications between peers will significantly improve user and business application availability. The SAN architecture should also support in providing a smoother, more consistent service by adding load balancing to the high availability and performance already mentioned.

# Conclusion

Many companies and organizations will evolve towards SAN technology without realizing it as new disk arrays, offline storage devices, servers and software that are all made fiber and SAN attachable. Some other organizations will adopt SAN aggressively or insistently to gain the potential benefits of far better utilization of resources, new level of availability, performance and the ability to better share corporate data across their organization. For ‘pioneers’ that adopt Storage Area Network, they must select proven or trusted suppliers with a pedigree or good reputation for producing reliable hardware or software. These ‘pioneers’ also must advance aggressively to gain benefits of a SAN but with more proper planning. Management tools and utilities should be distributed early to help control the SAN.

Many experts say that SAN will take several years to established during which many users will see these ‘pioneers’ work through an evolution from ‘SCSI-based’ to an environment with great availability and performance for global applications and globally shared data. Nevertheless, the key to success is the establishment of a ‘storage-centric’ environment supported by a high performance low latency fiber fabric that provides users with highly available access to clusters of application servers with many-to-many connectivity to shared online and offline storage.

# Appendixes

# References / bibliography

- Peripheral Concepts, Inc. – Storage Area Network white paper by Farid Neema

http://www.periconcepts.com/WhitePaper/Storage%20Area%20Networking%20(SAN).html

- ZDNET: eWeek: SAN and deliver

http://www.zdnet.com/eweek/stories/general/0,11011,2342638,00.html

- ZDNET India – Tech Zone - Seeking a solution in Storage Area Networks

http://www.zdnetindia.com/techzone/networking/stories/17862.html

- The Tech-PC Webopedia.com – SAN

http://thetech.pcwebopedia.com/TERM/S/SAN.html

- Storage Area Network

http://backofficesystems.com/tips/network/san.htm

- SAN – a whatis definition – see also: Storage Area Network

http://whatis.techtarget.com/definition/0,289893,sid9_gci212937,00.html

- SNIA – A dictionary of Storage Networking Terminology (letter S)

http://www.snia.org/English/Resources/Dictionary/Dictionary_S.html

- Introduction to Storage Area Networks – InfraStor Technologies Corp.

http://www.infrastor.com/tech/SANTechIntro.htm

- NetworkMagazine.com – Storage Area Network

http://www.networkmagazine.com/article/NMG20000724S0006

- Q Associates – What is SAN

http://www.qassociates.co.uk/storage/san/what-is-san.htm

- Storage Area Networks Capabilities – InfraStor Technologies Corp.

http://www.infrastor.com/tech/capabilities.htm

- Gadzoox – Storage Area Networking (SAN) Quick Guide by Dave Tang

http://www.gadzoox.com/san_library/quickguide.html